# Power-Line Noise Testbench — Simulink v06

## Purpose

This package provides the v06 Simulink power-line-noise testbench and three standard complex-IQ test signals. It supports viewing one-cycle magnitude traces, displaying a 60-cycle waterfall, and opening saved test recordings for comparison.

## Package contents

- powerline_cycle_waterfall_milestone_06.slx — v06 Simulink model

- initializeV06_script.m — loads the required signals and workspace variables

- openFieldEventWaterfall.m — opens a saved IQ recording in the waterfall display

- showCycleWaterfall.m — creates the cycle-synchronous waterfall

- waterfallClick.m — opens the magnitude trace for a selected waterfall cycle

- test_signals — folder containing the standard test cases

## Included standard test cases

- WN_ONLY_001.mat — complex white noise only

- SPARK_ONLY_001.mat — synthetic power-line sparking only

- SPARK_NOISE_EQUAL_RMS_001.mat — synthetic sparking and white noise with equal whole-record RMS contributions

Each test file includes the primary compatibility variables:

- eventIQ — complex single-precision IQ samples

- fs — sample rate in samples per second

- fc — nominal center frequency in hertz

Additional metadata describes the test case and its generation settings.

## Installation

- Download the ZIP package.

- Extract the complete ZIP to a normal writable folder.

- Do not run the model from inside the ZIP archive.

- Start MATLAB.

- Set the MATLAB Current Folder to the extracted package folder.

Keep the test_signals folder beside initializeV06_script.m.

## Startup

In the MATLAB Command Window, run:

initializeV06_script

Successful initialization reports:

- 2,400,000 samples

- 2,400,000 samples per second

It creates the workspace variables required by the model:

- sparkOnlyIQ

- whiteNoiseIQ

- gridReference

- fs

- fc

Then open:

powerline_cycle_waterfall_milestone_06.slx

Run the model from the Simulink toolstrip.

## Opening the standard test signals

- In the v06 model, click Open Field Recording.

- Browse to the included test_signals folder.

- Select one of the three MAT-files.

- Open the file to display its cycle-synchronous waterfall.

- Click a waterfall cycle to open its one-cycle magnitude trace.

Expected observations:

- White noise only: random background without repeating grid-aligned spark concentrations.

- Sparking only: repeatable impulse groups at consistent positions within the 60 Hz cycle; the strongest peaks are approximately 0.25 magnitude.

- Equal-RMS mixture: continuous noise background with repeatable spark groups; normalization permits peaks approaching 0.95 without intentional clipping.

## Important operating notes

- Run initializeV06_script.m before running the model.

- Keep all supplied helper files in the package folder.

- Do not rename variables inside the supplied MAT-files.

- Do not overwrite the supplied standard test files. Save experimental results under new filenames.

- The equal-RMS case uses whole-record RMS equality. It is an initial interface test and is not yet a formal event-window SNR validation case.

## Troubleshooting

### Undefined variable such as sparkOnlyIQ, whiteNoiseIQ, gridReference, or fs

Set MATLAB's Current Folder to the extracted package folder and rerun:

initializeV06_script

### Test-signal folder not found

Confirm that the extracted folder structure contains:

powerline_v06_student_package

├── initializeV06_script.m

├── powerline_cycle_waterfall_milestone_06.slx

├── openFieldEventWaterfall.m

├── showCycleWaterfall.m

├── waterfallClick.m

└── test_signals

├── WN_ONLY_001.mat

├── SPARK_ONLY_001.mat

└── SPARK_NOISE_EQUAL_RMS_001.mat

### Open Field Recording cannot find the desired file

Use the file dialog address controls to navigate into the extracted package's test_signals folder.

### Helper function not found

Confirm that the three helper M-files remain beside the Simulink model and that the package folder is MATLAB's Current Folder.

## Version

- Package: Power-Line Noise Testbench v06

- Prepared: September 27, 2026

- MATLAB/Simulink development environment: R2026a / Simulink 26.1